import time

import requests

import adafruit_dht

import board

import RPi.GPIO as GPIO

from RPLCD.i2c import CharLCD

DHT_SENSOR = adafruit_dht.DHT11(board.D4)

RAIN_SENSOR_PIN = 17

THINGSPEAK_API_KEY = '771CWSP58QHWY3BG'

THINGSPEAK_URL = 'https://api.thingspeak.com/update'

GPIO.setmode(GPIO.BCM)

GPIO.setup(RAIN_SENSOR_PIN, GPIO.IN)

lcd = CharLCD('PCF8574', 0x27, cols=16, rows=2)

while True:

try:

temperature = DHT_SENSOR.temperature

humidity = DHT_SENSOR.humidity

rain_status = GPIO.input(RAIN_SENSOR_PIN)

rain_text = "Rain" if rain_status == 0 else "No Rain"

print(f"Temp: {temperature}")

print(f"Humidity: {humidity} %")

print(f"Rain: {rain_text}")

lcd.clear()

lcd.write_string(f"T:{temperature}C H:{humidity}%")

lcd.crlf()

lcd.write_string(rain_text)

payload = {

'api_key': THINGSPEAK_API_KEY,

'field1': temperature,

'field2': humidity,

'field3': 1 if rain_status == 0 else 0

}

response = requests.get(THINGSPEAK_URL, params=payload)

print("Data sent to ThingSpeak Cloud:", response.status_code)

except Exception as e:

print("Error:", e)

time.sleep(15)